AND SMS INCIDENT REPORT

| INCIDENT DETAILS |  |  |
| --- | --- | --- |
| Date and time of the incident | : | 03 August 2024 |
| Incident Severity Level | : | Major |
| Incident Description | : | BRAC Bank clients received delayed SMS from GP. |

AFFECTED SYSTEM

Our SMS platform server partially lost connectivity with Infozillion. One of our paths was down due to an asymmetric route for upload and download paths, and the other paths were congested for this reason. Mainly, the Grameenphone’s single SMS request path was affected, which caused a long queue at the third-party end (ADN, Infozillion, Grameenphone).

IMPACT

Our standard DLR successful SMS detection mechanism failed to identify and filter out the successful SMS list due to the huge queue and incorrect response and timestamp from Infozillion and GP caused by the path congestion and down issue. As a result, our GP customers of BRAC Bank received delayed SMS and got a successful status from us through the API.

ROOT CAUSE ANALYSIS

Our technical team conducted a thorough investigation and identified that the root cause of the issue was related to our SMS application server partially losing connectivity with Infozillion. One of our paths was down due to an asymmetric route for upload and download paths. Initially, we found an issue at the Grameenphone end also, where there was a high queue. After communicating with them, they resolved the issue.

IMMEDIATE ACTIONS TAKEN

Upon identifying the issue, our technical team took immediate steps to mitigate the problem and ensure it does not recur. The following actions have been implemented:

1.   Path Configuration Update:

We have reconfigured and updated the network configuration to ensure optimal handling of concurrent processes. The upload and download paths were previously asymmetric (different), but we have now changed them to be symmetric (using the same path).

2.   Enhanced Monitoring and Alerts:

ADN Telecom has deployed advanced network and path congestion status monitoring tools to detect any anomalies in real time, coupled with automated alerts to promptly address any potential issues.

3.   Path Management Logic Enhancement:

The path management processing logic has been enhanced to include additional checks and validations to prevent these types of issues.

4.   Redundancy Path and Failover Mechanism:

We have implemented additional redundancy path and failover mechanisms to ensure

uninterrupted service.

LONG TERM REMEDIATION

In addition to the immediate actions, we are committed to continuous improvement of our systems. We are working on the following long-term enhancements:

1.   Advanced path shifting Algorithms:

We are working on developing more sophisticated algorithms for detecting and preventing path-down issues, along with automatic shifting to redundant and tertiary paths.

2.   System Audits and Stress Testing:

Regular system audits and stress testing to identify and rectify any potential weaknesses in

our infrastructure.

3.   Comprehensive Training for Technical Staff:

Ongoing training programs for our technical staff to ensure they are equipped with the latest

knowledge and skills to manage and troubleshoot such issues effectively.

INCIDENT RESPONSE TEAM

| Sohael Reza | Monitored and guided the Incident Response Team. |
| --- | --- |
| Shameem Hasan | Coordinated with internal and external teams. |
| Mizanur Rahman | Followed up on the Incident Management Process. |
| Mukaddass Ali | Aligned with the internal KAM team and customers. |
| A.K.M. Nurul Hoque | Stood by for application end issues. |
| Amir Hossain (DevOPS) | Closely monitored the server. |
| TechOPS | Raised the incident and followed up. |

COMMUNICATION

The ADN Team (Internal) followed the Incident Management process.

LESSONS LEARNED

Teamwork, proper communication, and process compliance.

INCIDENT CLOSURE

A malfunction occurred where our SMS platform server partially lost connectivity with Infozillion. One of our paths was down. The upload and download paths were previously asymmetric (different), but we have now changed them to be symmetric (using the same path).